GDSReady-RV

RTL-to-GDSII Implementation of a 32-bit 5-Stage Pipelined RISC-V Processor

Final Project Report
ASIC implementation using LibreLane and the SKY130 HD standard-cell flow

| Design | riscv_core_asic |
| --- | --- |
| Architecture | 32-bit, 5-stage pipelined RISC-V |
| Pipeline | IF → ID → EX → MEM → WB |
| Clock target | 10.0 ns (100 MHz) |
| Final physical run | RISCV_ASIC_PNR_01 |

# 1. Abstract

This project implements a 32-bit RISC-V-style processor as a complete RTL-to-GDSII flow. The processor uses a five-stage IF/ID/EX/MEM/WB pipeline, a 32-entry register file, an ALU supporting the required arithmetic and logical operations, immediate generation, control and branch logic, forwarding, load-use hazard detection, instruction/data memory for RTL verification, and an ASIC-facing memory interface for physical implementation. The RTL was verified using directed regression tests for ALU operations, forwarding, load/store behavior, load-use hazards, BEQ/BNE, and JAL. The ASIC-facing core was synthesized and taken through floorplanning, placement, clock-tree synthesis, routing, post-route timing, DRC, LVS, antenna checking, and GDSII generation.

# 2. Objectives

Develop a synthesizable 32-bit RISC-V processor with a five-stage pipeline.

Implement register-file, ALU, immediate, control, branch, forwarding, and hazard-management logic.

Verify representative RV32I-style instruction and pipeline behaviors using simulation.

Create an ASIC-facing top-level interface for instruction and data memory.

Complete synthesis and physical implementation using LibreLane/OpenROAD and SKY130 HD standard cells.

Generate measurable implementation results including area, utilization, timing, DRC, LVS, antenna, and GDSII evidence.

# 3. Architecture

The processor follows the five-stage pipeline specified for the project: Instruction Fetch (IF), Instruction Decode (ID), Execute (EX), Memory Access (MEM), and Write Back (WB). Pipeline registers separate the stages: IF/ID, ID/EX, EX/MEM, and MEM/WB.

Program counter and instruction-fetch path

Instruction decoder and control unit

Immediate generator

32 × 32-bit register file with x0 hard-wired to zero

ALU supporting ADD, SUB, AND, OR, XOR, SLT, SLL, SRL, and SRA

Branch/jump target and decision logic

Forwarding unit for EX/MEM and MEM/WB dependencies

Hazard detection for load-use dependencies

Instruction and data memory for RTL simulation

ASIC-facing instruction/data memory ports

# 4. Pipeline and Hazard Handling

## 4.1 Forwarding

The forwarding unit provides EX/MEM-to-EX and MEM/WB-to-EX operand forwarding. This permits dependent arithmetic instructions to execute without unnecessary pipeline stalls.

## 4.2 Load-use hazard

A load followed immediately by an instruction consuming the loaded register is detected by the hazard unit. The verified regression demonstrates the required stall behavior.

## 4.3 Branch and jump handling

BEQ and BNE branch decisions are supported by the branch unit. JAL generates the jump target and writes PC+4 to the destination register. Both branch behavior and JAL were explicitly verified.

# 5. RTL Module Structure

alu.s

register_file.s

program_counter.s

immediate_generator.s

instruction_decoder.s

control_unit.s

branch_unit.s

forwarding_unit.s

hazard_detection.s

instruction_memory.s

data_memory.s

if_id_reg.s

id_ex_reg.s

ex_mem_reg.s

mem_wb_reg.s

riscv_core.s

riscv_core_asic.s

# 6. RTL Verification

The final directed RTL regression completed successfully. The simulation produced only the expected $readmemh short-file warnings because the test programs contain fewer than the 256 initialized memory words; these warnings did not cause test failures.

| Test | Key results | Purpose | Result | Evidence |
| --- | --- | --- | --- | --- |
| ALU + forwarding | x1=5, x2=10, x3=15, x4=0xFFFFFFFB, x5=10 | RAW dependencies and ALU | PASS | final_regression_results.txt |
| Load/store + load-use | x3=1, x4=9, memory[2]=1 | Memory path and load-use stall | PASS | final_regression_results.txt |
| BEQ/BNE | x3=7, x6=9 | Taken/not-taken branch behavior | PASS | final_regression_results.txt |
| JAL | x1=4, x2=7, x3=5 | Jump target and PC+4 writeback | PASS | final_regression_results.txt |

Final RTL regression result: ALL RTL REGRESSION TESTS PASSED.

# 7. ASIC-Oriented RTL Architecture

The initial RTL top-level used internal instruction/data memories. For ASIC implementation, an explicit riscv_core_asic top-level was introduced with instruction-memory and data-memory interfaces. This avoids relying on behavioral memory arrays as the physical-design boundary and produces a real standard-cell implementation with external memory ports.

# 8. RTL-to-GDSII Flow

RTL elaboration and synthesis with Yosys through LibreLane

Floorplanning and standard-cell row generation

Placement

Clock-tree synthesis

Global and detailed routing

Post-route static timing analysis

Antenna checking and repair

Magic DRC

KLayout DRC

Netgen LVS

Final GDSII stream-out

# 9. Synthesis Results

| Metric | RISCV_ASIC_SYNTH_02 |
| --- | --- |
| Mapped standard-cell instances | 7,070 |
| Mapped instance area | 86,360.3264 |
| Unmapped instances | 0 |
| Inferred latches | 0 |
| Synthesis check errors | 0 |
| Lint errors | 0 |
| Lint warnings | 2 |

The synthesis run produced a full ASIC-facing mapped netlist. The netlist contains memory interface ports and thousands of mapped standard-cell instances, unlike the earlier internal-memory top-level attempt that optimized away most of the processor logic.

# 10. Physical Implementation Results

| Metric | RISCV_ASIC_PNR_01 |
| --- | --- |
| Clock target | 10.0 ns / 100 MHz |
| Post-P&R utilization | 47.3588% |
| Post-P&R aggregate setup WNS | −7.232224 ns |
| Post-P&R aggregate setup TNS | −1688.802674 ns |
| Hold WNS | 0 ns |
| Hold TNS | 0 ns |
| Worst nom_ss post-P&R setup WNS | −6.861401 ns |

The final physical implementation is not setup-timing-clean at the 10 ns target under the slow-corner analysis. The worst post-P&R nominal slow-corner setup WNS is −6.861401 ns at 100 °C and 1.60 V. Hold timing is clean in the reported corners. The negative setup slack is retained as an implementation result rather than being presented as timing closure.

# 11. Physical Verification

| Check | Result | Evidence |
| --- | --- | --- |
| OpenROAD detailed-routing DRC | PASS / no final violations | riscv_core_asic.drc |
| Magic DRC | PASS | drc.magic.rpt |
| KLayout DRC | PASS; total = 0 | drc.klayout.json |
| Netgen LVS | PASS; circuits match uniquely | lvs.netgen.rpt |
| Antenna | PASS; no violating entries | antenna_summary.rpt |

# 12. Final GDSII

The final physical implementation was streamed out as a GDSII database. The packaged deliverable is riscv_core_asic_final.gds. The final package also contains the corresponding LEF and DEF, synthesized netlist, timing reports, DRC reports, LVS report, antenna report, and metrics.

final/gds/riscv_core_asic_final.gds

final/layout/riscv_core_asic.lef

final/layout/riscv_core_asic.def

final/netlist/riscv_core_asic.nl.v

# 13. Implementation Iterations

| Iteration | Configuration | Observed result | Decision |
| --- | --- | --- | --- |
| Initial synthesis | riscv_core with internal memories | CPU optimized away; floorplan had no standard-cell rows | Rejected |
| RISCV_SYNTH_02 | Debug outputs added to internal-memory top | 162 cells; incomplete CPU preserved | Rejected for P&R |
| RISCV_ASIC_SYNTH_02 | Explicit ASIC memory ports | 7,070 mapped cells; area 86,360.3264 | Used as synthesis baseline |
| RISCV_ASIC_PNR_01 | 40% floorplan utilization target | 47.36% final utilization; physical checks passed; −6.861 ns worst post-P&R setup WNS | Primary final run |
| RISCV_ASIC_PNR_02 | 35% floorplan utilization / 42% target density | Physical checks passed but setup WNS degraded to about −8.396 ns aggregate | Kept as experiment |

# 14. Timing Investigation

The worst timing cone was traced to logic associated with the MEM/WB control and writeback path feeding the ID-stage register-file read/bypass network. The critical path investigation identified synthesized registers and deep combinational mux/logic cones between pipeline state and ID/EX register inputs. Several RTL restructuring experiments were evaluated, but changes that altered the verified register-file write-through behavior caused RTL regression failures. A cosmetic bypass-expression restructuring produced an identical synthesis result. The verified RTL was therefore restored and frozen before final reporting.

# 15. Limitations and Engineering Observations

The final 10 ns target is not met for setup timing at the slow corner.

Hold timing is clean in the reported signoff corners.

The current implementation uses an ASIC-facing external memory interface rather than a compiled SRAM macro.

The RTL regression uses behavioral memories for functional verification.

The project evidence does not contain a reliable final power number; power is therefore not reported as a measured final metric.

The implementation uses SKY130 HD standard cells and is an academic/open-source RTL-to-GDSII demonstration rather than a tapeout-qualified commercial ASIC.

# 16. Conclusion

The project demonstrates a complete RTL-to-GDSII implementation of a 32-bit five-stage pipelined RISC-V processor. Functional RTL verification passed all four final directed regression groups covering ALU/forwarding, load-store/load-use hazards, BEQ/BNE, and JAL. The ASIC-facing design was synthesized into a 7,070-cell mapped standard-cell netlist and successfully passed physical DRC, LVS, and antenna checks. A final GDSII database was generated together with LEF/DEF and signoff evidence. The principal remaining implementation limitation is setup timing at the 10 ns target under the slow corner, while hold timing remains clean.

# 17. Final Deliverables

GDSII: final/gds/riscv_core_asic_final.gds

LEF: final/layout/riscv_core_asic.lef

DEF: final/layout/riscv_core_asic.def

Synthesized netlist: final/netlist/riscv_core_asic.nl.v

RTL sources: final/rtl/

RTL regression: final/reports/final_regression_results.txt

Synthesis metrics: final/reports/synthesis_metrics.csv

P&R metrics: final/reports/metrics.csv and metrics.json

Setup timing: final/reports/setup_nom_ss_max.rpt

Hold timing: final/reports/hold_nom_ss_min.rpt

Magic DRC: final/reports/drc.magic.rpt

KLayout DRC: final/reports/drc.klayout.json

LVS: final/reports/lvs.netgen.rpt

Antenna: final/reports/antenna_summary.rpt

# Appendix A — Final RTL Regression Output

The archived regression reports PASS for all four directed tests. The simulator also reports expected $readmemh warnings because the hex files are shorter than the initialized 256-word memory range.

ALU + FORWARDING: PASS

LOAD/STORE + LOAD-USE: PASS

BEQ/BNE: PASS

JAL: PASS

FINAL RTL REGRESSION RESULT: ALL RTL REGRESSION TESTS PASSED